Supplementary information

Comparison of laboratory scale bourbon whiskey mashing and fermentation to pilot scale

Table S1. Particle size distribution for grains used at the lab-scale.

| Grain Type | Percent retained on sieve |  |  |  |  |  |  |  | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2380 μm | 1180 μm | 425 μm | 300 μm | 250 μm | 180 μm | 75 μm | Pan |  |
| Corn | 0% | 11% | 67% | 5% | 3% | 10% | 3% | 0% | 100% |
| Rye | 0% | 16% | 55% | 8% | 3% | 13% | 4% | 1% | 100% |
| Barley Malt | 1% | 21% | 47% | 8% | 2% | 7% | 9% | 4% | 100% |

Table S2. Rapid visco analyser (RVA) individual grains temperature profile

| Time | Type | Value |
| --- | --- | --- |
| 00:00 | Temp. | 50°C |
| 00:00 | Speed | 960 RPM |
| 00:10 | Speed | 160 RPM |
| 01:00 | Temp. | 50°C |
| 04:42 | Temp. | 95°C or 122°C |
| 7.12 | Temp. | 95°C or 122°C |
| 11:00 | Temp. | 50°C |
| End of test time: 13:00 (mm:ss) |  |  |
| Initial idle temperature: 50 ± 2°C |  |  |
| Time between readings: 4 s |  |  |
| Viscosity units: cP (≙ mPas) |  |  |

Table S3. Rapid visco analyser (RVA) Mash Temperature Profiles

| Single Bourbon mash conversion process (Simultaneous Saccharification and Fermentation [SSF]) |  |  | Multi-step Bourbon mash conversion process (conventional mash) |  |  |
| --- | --- | --- | --- | --- | --- |
| Time | Type | Speed | Time | Type | Value |
| 00:00 | Temp. | 25°C | Add 6.83g Corn |  |  |
| 00:00 | Speed | 960 RPM | 00:00 | Temp. | 93°C |
| 00:10 | Speed | 160 RPM | 00:00 | Speed | 960 RPM |
| 10:00 | Temp. | 122°C | 00:10 | Speed | 160 RPM |
| 1:00:00 | End |  | 01:00 | Temp. | 93°C |
| Add 1.5uL enzyme mix |  |  | 30:00 | Temp. | 93°C |
| 00:00 | Temp. | 30°C | 35:00 | Temp. | 77°C |
| 00:00 | Speed | 960 RPM | 35:00 | End |  |
| 00:10 | Speed | 160 RPM | Add 1.37g Rye |  |  |
| 30:00 | End |  | 00:00 | Temp. | 77°C |
|  |  |  | 00:00 | Speed | 960 RPM |
|  |  |  | 00:10 | Speed | 160 RPM |
|  |  |  | 01:00 | Temp. | 77°C |
|  |  |  | 10:00 | Temp. | 77°C |
|  |  |  | 15:00 | Temp. | 63°C |
|  |  |  | 15:00 | End |  |
|  |  |  | Add 0.911g Malt |  |  |
|  |  |  | 00:00 | Temp. | 63°C |
|  |  |  | 00:00 | Speed | 960 RPM |
|  |  |  | 00:10 | Speed | 160 RPM |
|  |  |  | 15:00 | Temp. | 63°C |
|  |  |  | 25:00 | Temp. | 30°C |
|  |  |  | 25:00 | End |  |
|  |  |  | Add 1.5uL enzyme mix |  |  |
|  |  |  | 00:00 | Temp. | 30°C |
|  |  |  | 00:00 | Speed | 960 RPM |
|  |  |  | 00:10 | Speed | 160 RPM |
|  |  |  | 30:00 | End |  |

Figure S4. Gas production (solid lines) and gas production flow (dashed lines) for lab-scale conventional (Conv) with and without added enzymes and Simultaneous Saccharification and Fermentation (SSF) mashing methods.